Whole genome sequencing notes – Antonella Eugenia Bacigalupo – 25/2/25

Antonella is willing to show protocol over and assist if Martin is ok with. Antonella begins as tech in March so will be more available after her thesis is completed.

Below is augmented note summary from brief meeting

To do 
Determine genome size to ensure feasibility.

Extract high-quality, long-fragment DNA with modified protocols.
Assess DNA integrity before sequencing.
Use Oxford Nanopore for long reads, but consider hybrid assemblies with Illumina for accuracy.
Address mtDNA challenges with alternative approaches if needed.

Whole Genome Sequencing (WGS) of Bryozoan and Hydrozoan Species Using Oxford Nanopore

1. Genome Size Assessment

Before proceeding with sequencing, it is essential to determine the genome size of each species. This helps in:

- Assessing feasibility – Some genomes may be too large for efficient sequencing with Oxford Nanopore.

- Estimating sequencing depth – Larger genomes require more sequencing data to achieve sufficient coverage.

- Planning data storage & analysis – Larger datasets require more computational power for assembly and analysis.

Genome size can be estimated using flow cytometry, k-mer analysis from short-read sequencing, or Feulgen staining.

2. DNA Quality and Fragment Length

Oxford Nanopore sequencing relies on long-read sequencing technology, meaning that the quality and length of input DNA are critical. High molecular weight (HMW) DNA is essential because:

- Longer fragments improve assembly accuracy and allow sequencing through repetitive or complex regions.

- Highly fragmented DNA reduces read length and impacts the ability to generate contiguous genome assemblies.

- Heterogeneous species (those with significant genetic variation) can complicate genome assembly due to mixed haplotypes, making high-quality, high-coverage sequencing even more important.

Clonal sequencing (using clones of the same DNA) is preferable because it eliminates genetic variability, making assembly easier and more reliable.

3. Assessing DNA Fragment Size and Quality

To determine whether extracted DNA is suitable for sequencing, various techniques should be used:

- Capillary electrophoresis (e.g., Bioanalyzer, Fragment Analyzer) – Provides an accurate fragment size distribution.

- TapeStation – Assesses if DNA is fragmented or intact.

- Qubit Fluorometry – Measures DNA concentration without being affected by small degraded fragments (unlike Nanodrop).

- Nanodrop Spectrophotometry – Assesses purity (260/280 and 260/230 ratios) but is unreliable for quantification.

For long-read sequencing, DNA should ideally have a fragment size >50 kb, with N50 values >30 kb being desirable.

4. DNA Extraction and Optimization

DNA extraction using the DNeasy Qiagen Blood & Tissue Kit requires modifications to preserve long fragments and improve sequencing performance:

- Buffer Modification – Use TE buffer instead of the standard elution buffer to protect DNA from degradation.

- Minimizing Shear Forces – Avoid vortexing, bead beating, and high-speed pipetting to prevent fragmentation.

- Pre-heating Filtration Tubes – Warming to 37°C before final extraction can improve elution efficiency without breaking DNA strands.

- Optional: Phenol-Chloroform Extraction – Can be used for higher-purity, ultra-HMW DNA but requires additional handling.

Additional considerations:

- Using agarose gel size selection can remove small fragments before sequencing.

- Enzymatic cleanup (e.g., RNase treatment) removes unwanted RNA contamination.

5. Oxford Nanopore Sequencing Workflow

Once high-quality DNA is obtained, the Oxford Nanopore sequencing process follows these steps:

- Library Preparation – Requires minimal fragmentation and ligation of adapters for compatibility with the flow cell.

- Loading onto a Flow Cell – Read length and throughput depend on pore occupancy and DNA integrity.

- Real-Time Sequencing – Can last from one day to a week, depending on sequencing goals and quality.

- Basecalling & Quality Control – Using Guppy or ONT’s Bonito basecaller for improved accuracy.

- Assembly & Error Correction – Hybrid assemblies (Nanopore + Illumina) improve accuracy, reducing high Nanopore error rates.

Common challenges:

- Flow cell clogging – Due to DNA shearing or contaminants.

- Read quality variability – Especially for repetitive or GC-rich regions.

- Basecalling accuracy – Current improvements in ONT chemistry and AI-based basecalling are addressing this issue.

6. Mitochondrial DNA (mtDNA) Considerations

One key consideration is whether the mtDNA is being properly captured during sequencing.

- mtDNA is typically smaller (16-30 kb) and circular, making it less likely to be enriched in long-read libraries.

- If targeting mtDNA specifically, alternative approaches may be needed:

  - Long-range PCR before WGS to amplify mtDNA.

  - Illumina sequencing – More cost-effective for targeted mtDNA sequencing.

For environmental DNA (eDNA) studies, designing WGS-compatible primers to target mitochondrial regions can be challenging, as Oxford Nanopore may not consistently capture mtDNA due to fragmentation biases.

7. Alternative Approaches

- Illumina short-read sequencing can complement Nanopore for hybrid assemblies and improve accuracy.

- PacBio HiFi sequencing may offer higher accuracy for difficult genomes but is more expensive.

- Targeted enrichment (e.g., Cas9-based enrichment) can selectively sequence mitochondrial or specific genomic regions.